AT2424SAL Jumpers

(Ascii Communications Format Version)

Revised 17 Oct 2003

R=Jumper Removed, I=Jumper Installed

***=Factory Default

J7-J9:		Retry Time

J7	J8	J9	Retry Time

I	I	I	10 Seconds***

I	I	R	5 Seconds

I	R	I	2 Seconds

I	R	R	1 Second

R	I	I	500 milliseconds

R	I	R	200 milliseconds

R	R	I	100 milliseconds

R	R	R	50 milliseconds

J10:		Diagnostic1 	I=Copy Inputs 1-24 to Outputs 1-24, R=Do Not Copy to Outputs***

J11:		Diagnostic2	I=Copy J7-J24 to Outputs 7-24, R=Do Not Copy to Outputs***

J12-J15		Not Used

J16:		Master/Slave Mode.	I=Master***, R=Slave

Note: All units are shipped as “Master”. One of the pair of units linked together should be set by the user as “Slave”(Remove J16).

J17:		Check Sum Control.	I=Enable Check Sum Test on Received Data***, R=Disable

J18  & J19:	Drop Out Time

J18	J19	Drop Out Time

I	I	10 Seconds***

I	R	5 Second

R	I	2 Second

R	R	Never

(Jumpers 21 thru 24 on next page)

J20-J23		Baud Selection

J20	J21	J22	J23	Baud

I	I	I	I	9600***

I	I	I	R	115200

I	I	R	I	9600

I	I	R	R	57600

I	R	I	I	38400

I	R	I	R	28800

I	R	R	I	19200

I	R	R	R	14400

R	I	I	I	9600

R	I	I	R	4800

R	I	R	I	2400

R	I	R	R	1200

R	R	I	I	600

R	R	I	R	9600

R	R	R	I	9600

R	R	R	R	9600

J24:		Not Used

AT2424SAL Data Format

Revised 10 Apr 2003

Note: This structure is for point to point communications between one master and one slave I/O board. It does not include an address field.

## <M/S><Format><Flags><-><Data(2-6)><*><CheckSum(4)><CRLF(2)>

## <M/S>		Master/Slave tag. Tags the data so the display board knows where to put it.

<Format> 	Single character 0 to F. Allows for future data formats as they are dreamed up.

0 = only currently defined format.

The balance of the packet depends upon the format defined by the Format byte above.

Format 0:

<Flags> 	Single character 0 to F to define 4 flag bits.

Bit		Definition

- Previous received packet error status. 0=OK,

`	2                   Timeout Error. No response from slave

- Reserved

### 0	           Reserved

<->		Dash. Start of data field marker

<Data(2-6)>	Data field. 2 to 6 Ascii Hex Data nibbles describing 8 to 24 bits of I/O data.

<*>                 Asterisk. End of data field marker.

<CheckSum(4)>	16 bit two’s complement of all characters up to, and including, the end of data  field marker.

<CRLF(2)>	Two characters,  D(hex) and A(hex)